##### 3.01	SOP FOR CARE, MAINTENANCE AND CLEANING OF ARMS

3.01.1         Purpose

The purpose of this document is to lay down the SOP for the maintenance and cleaning of arms held at CSCR Jamnagar.

3.01.2        Scope

The scope of this document covers-

a. The 0.315 bore Rifles kept at CSCR Jamnagar.

b. The 0.32 bore Revolvers kept at CSCR Jamnagar.

3.01.3         Type of maintenance

The maintenance can be of following types:

- a.  Deliberate cleaning.

b.  Weekly cleaning.

c.  Cleaning before firing.

d.  Cleaning after firing.

Note: Make sure that the weapon is unloaded before any cleaning/ servicing.

      - Deliberate cleaning

Deliberate cleaning of arms is carried out before depositing the arms to CSCR after patrolling/exercise or bonafide duty by the individual using the weapon. In this type of cleaning dust and humidity is removed from the arms by cleaning it with dry flannelette and then applying a thin layer of gun oil.

      - Weekly cleaning

This shall be carried out every week under the supervision of Duty Officer CSCR in following manner:

        - Items Required

- Cleaning Rod/Pull through

- Lubricating oil.

- Flannelette

###### 3.01.3.3   Procedure for Rifle

- Remove the dust from the arm.

- Remove magazine.

- Bring safety catch to forward position and then remove the bolt.

- Clean the barrel with dry flannelette repeatedly.

- Lubricate the barrel with gun oil

- Apply thin layer of gun oil on the bolt

- Assemble the bolt to rifle, move it to and fro and be sure of correct functioning of the arm

- Bring the safety catch to backward position

- Keep the rifle on the rack

3.01.3.3.1	Method for cleaning the Rifle.

First of all clean the rod by applying slight oil on it. Put the flannelette in the eye of the rod for cleaning. Keep the rifle on the table in such a manner that the foresight and back sight are facing upwards and the trigger guard is firmly in contact with the edge of the table. Hold the reinforced portion of the barrel (just behind the backsight) with the left hand firmly. Insert the cleaning rod with flannelette through breech end gently upto the muzzle end and pull it back

Repeat the to and fro motion several times to make sure that the bore/barrel is clean.

Note: The components of one rifle should not interchange with others.

Note:  The wooden part of the rifle shall be preserved with linseed raw oil.

###### 3.01.3.4  	Procedure for Revolver

- Keep the revolver on the table and clean it with dry flannelette to remove dust

- Break open the revolver by pressing the barrel catch with right hand thumb and pressing the barrel by left hand

- Clean and lubricate the barrel with gun oil. The following method (as given in para 3.01.3.4.1) shall be followed for cleaning the revolver.

- Assemble the weapon and check for correct functioning. Press the     safety catch to safe position

3.01.3.4.1	Method for cleaning the Revolver.

First of all clean the rod by applying slight oil on it. Put the flannelette in the eye of the rod for cleaning. Every chamber shall be cleaned separately. Thereafter clean the barrel with the help of cleaning rod. Ensure that all the chambers and the barrel are clean. After cleaning, lubricate the chambers in the cylinder.

3.01.3.5          Cleaning before firing

- Clean the barrel and other part of rifle with dry flannelette to remove lubricating oil

- Check the correct functioning of arm.

3.01.3.6           Cleaning after firing

- Items required

- Flannelette

- Cleaning rod/pull through

- Hot water

- Funnel.

3.01.3.6.1	Procedure

- Keep the barrel in vertical position with muzzle downward

- Place funnel on the breach end of the barrel and pour water into funnel so that hot water can wash out the carbon deposits inside the barrel

- Clean the barrel with dry flannelette

- Repeat the procedure several times till the barrel is clean

- Lubricate the barrel

- Follow this procedure for the next consecutive days till the sweating is over

3.01.4             Conclusion

Weapon is a trusted and dependable companion of a Security Officer.  Life of a weapon increases with periodic cleaning and it can function reliably if it is cared and maintained at regular intervals.