| Reliance Petroleum Limited Jamnagar Refinery Complex Project Procedure 22960-G69-000- 1211B ACCESS CONTROL SYSTEM – VEHICLES DESIGN BASIS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 01 | 20.09.99 |  | CD | RL |  |
| Rev | Date | Revision | By | Chkd | Appr |

# OBJECTIVE

The objective of this document is to provide the design basis for an effective Access Control of vehicles entering the Refinery Complex to ensure a smooth flow of authorised vehicles and their logging at various locations and prevent any unauthorized vehicle from gaining entry into the Refinery Complex.

# OVERVIEW

- The vehicles requiring access into the Refinery Complex can be categorized in the following manner.

Vehicle meant for

- Solid Receipt and Dispatch				-	Trucks

- Liquid Receipt and Dispatch			-	Trucks and Tankers

- Material Movement

- Material for Projects				-	Trucks,

- Equipment, etc					-	Trucks, Cranes, etc.

- Scrap						-	Trucks,

- Internal Movement				-	Trucks,

- Garbage Collection & Disposal		-	Trucks, Dumpers,

- Personnel Movement

- Company Owned Vehicles			-	Buses, Cars, Jeeps,

- Company Hired Vehicles			-	Buses, Cars, Jeeps,

- Private Vehicles

- Employees					-	Cars, Jeeps, 2-wheels,cycles

- Contractors & Vendors			-	Buses, 2-wheelers, cycles

- Visitors					-	Cars, Jeeps

- VIP Vehicles					-	Cars, Jeeps,

- Emergency Vehicles

- Security

- Fire & Safety

- Ambulances

# VEHICLE ACCESS CONTROL POINTS

The vehicles would be controlled at the following points –

Loop 1 – Refinery, Marine Area, Haul Road Township Main Entrance

- Symbolic Gate

- Project Gate

- Main Materials Gate

- MTF Main Gate

- Haul Road SH25 Gate

- Haul Road MTF side Gate

- Township Main Gate

Loop 2 – Critical Internal locations

- Refinery Gate

- Liquid Parking In

- Solid Parking In

- MTF Battery Limit Gate

Loop 3 – Other Internal Locations

- SSO Area

- Various other gate locations – specify as bullet points

# PHILOSOPHY (change headings as in 2 above and insert bullet points)

## SOLID RECEIPT AND DISPATCH

- All employees residing in Jamnagar would be brought to the Township Bus Terminal.

- Respective shift buses that would first pickup the township employees from their respective bus stops and then the employees coming from Jamnagar from the Bus Terminal.

- Buses will be driven by security drivers who will ensure that only authorized persons board the buses.

- All employees boarding buses would be required to show and log their Access Cards at Portable Card readers mounted inside the bus.

- The data stored in these Terminals would be transferred to the on-line Access Control system at the Refinery Gate.

- All employees will be encouraged to use only authorized buses for commuting. Very few employees will, in exceptional contingencies, come by their own / private vehicles. At the Main Gate, Material Gate and MTF Gate security will check their ID Cards and allow entry.

- Such employees requiring enter to the Battery Limit Area, shall park their vehicles at MAB parking and board the shuttle bus.

- No individual vehicles shall be allowed inside the Battery Limit Area except Shuttle Buses. Plant vehicles would remain within the B/L Area. Only Security and Fire vehicles and Ambulances shall be granted entry into B/L Area.

- Inside the Battery Limit Area there will be no individual access control for separate plants, but in certain critical areas e.g. RCB/ ACB/ PIBs etc. , access shall be controlled by using Smart Card Readers combined with PIN pad or Biometric Readers.

- All employees working on Ave J i.e. Liquid Loading area / Marketing Office / Rail Loading area/ Solid Parking area, etc. shall enter from the Material Gate on Ave J and will be dropped at their respective work areas.

- They will gain access to their areas using their Card at the turnstile locations.

- Employees requiring to cross over from non-process areas to Battery Limit Area during the course of the day will be required to pass through turnstiles using their Card.

## EXIT

- Employees shall board the respective buses from their plants / project area / SSO / MAB.

- Employees working on Ave J will have separate bus for being transported to Township.

- All employees would need to show and log their card in the Portable Card Readers while boarding the buses.

- The data is again transferred at the respective Main / Refinery / Materials / MTF gates.

- Employees who have to go to Jamnagar city shall de-board the bus at Township Bus Terminus and board the buses for city.

## THE VISITORS

- All visitors coming to Refinery from outside will report at the Reception at Main Gate / MTF Gate (separate procedure for visitors coming through Material Gate).

- Security shall ensure that none of the visitors goes beyond these gates without proper authority i.e. Visitor Entry Card.

- Visiting hours shall be normally during General Shift only.

- If mobile, the vehicle is parked in the transit Parking near the entry block.

- At Reception their identity is verified and after confirmation from the employee (HOD’s in case of entry required into B/L Area) whom the visitor wants to meet, a Visitor Entry Card (Single sector smart card) is registered as being issued to the visitor and handed over.

- Separate color coded and pre-programmed Cards will be kept ready to be used for the visitors' visiting different areas so as to prevent them from transgressing to unauthorised premises.

- Once the Visitor Entry Card is prepared, his baggage will be checked. Any objectionable / surplus / extra baggage will be deposited at the gate.  Lockers shall be provided for the visitors to keep their luggage, only to be released once they return back.

- The Visitor then passes through the Door Frame Metal Detector and Turnstile using his Card.

- He is then briefed about the plant safety rules and procedures using a Video Demonstration including precautions to be taken and procedures during emergency.

- As per his turn, he boards the shuttle bus to take him to SSO / MAB / Refinery Gate and Project area.

Visitors for Refinery Area from Main Gate

- If the visitor has the permission to enter the Battery Limits / Refinery Area, he will report to the Reception at Refinery gate.

- The relevant card issued from the Main Gate shall allow him to pass through Refinery Gate turnstile (Anti sabotage check against carrying match box/ lighter/ explosive substances shall be carried out at this point also).

- At the Refinery Gate the visitor will wait till an escort reaches the gate to take him in. After the entry, the visitor will be escorted to his destination.

- From the Refinery gate onwards all movements for visitors will be on Shuttle bus / department vehicles.

### Visitors entry from Material Gate

- Hectic activity is expected in this area and a number of agencies will be visiting here i.e. contractors / transporters / marketing agencies / vendors etc.

- These people are not required to visit into the process area.

- All visitors will report at Materials Gate (procedure same as the Main Gate).

- All visitors coming on their vehicles will be allowed to move in on their vehicles except three wheelers.

- Those coming on foot will use the shuttle service from Material gate to Road / Rail Loading areas.

- No visitors are permitted to enter the loading areas (solid / liquid).

- Small parking will be developed near liquid loading / rail terminal and solid parking for the visitors, transporters and Oil Company people  etc.

### Exit

- The visitor will get a signed confirmation from the executive he visited.

- He shall return his Visitor Entry Card at the time of exit at Main gate / Material gate / MTF Gate.

- At the Main Gate / MTF Gate, when the Visitor passes through the turnstile using his Card, it will be ensured that the Card is retained in a drop-in type of box.

- Random frisking / checking of material will be done.

- It will be ensured that no visitor crosses the Main gate in the shift buses, private vehicles, etc. without undergoing the required checks.

- None of the visitors will be allowed to travel in shift buses.

## CONTRACT WORKERS

A. Contract workers entry from Project Gate

- All contractor workers (approx. 2000), coming to work in Refinery / Project area will come from Projects Gate.

- They will enter the Workers’ marshalling area by showing their ID Cards which will have color coding for different areas.

- Workers for Workshop/ CMC/ other offices and those meant for Green belt will also be allowed from this gate.

- Initial Biometric registration of all Contract Workers shall be done at the Project Gate.

- Contract Workers requiring to enter the Project Area or Green Belt between Ave A and Project Gate will be required to pass through turnstiles using access card with Hand Geometry reader.

- Contract workers for the B/L Area will be transported by buses to the Contract Workers Gate on Street 1.

- They would pass through turnstiles using access card with Hand Geometry reader located at the Terminal.

B. Contract Workers entry from Construction Gate (LC-2)

- Construction workers working in Refinery site and staying in LC-2 (approx. 2000) and those working in the Green belt area on the western side of Ave - A inside Battery limit will enter from the LC-2 entrance i.e. Construction Gate.

- They would pass through turnstiles using access card with Hand Geometry reader.

C. Contract Workers entry from Main Gate

- Contractor workers for MAB / SSO complex (approx. 400) and the Green belt area on north-western side outside battery limit will enter through the Main Gate.

- They would pass through turnstiles using access card with Hand Geometry reader.

D. Contract Workers entry from Materials Gate

- Contract workers working in Rail / Road loading area and Green belt will be checked and allowed to enter through the Materials Gate.

- They will be given a different colour pass and will not be allowed in other areas inside the refinery unless specifically authorised.

- From the Materials Gate shuttle service will be used for them to be dropped at their work site.

- At the entrances of all the facilities on Ave J they will be frisked and checked and will pass through the turnstiles using Card and Hand Geometry Reader.

- All Contract workers, Oil company staff, Railway staff and transporters representatives' etc. will also pass through the same procedure.

E. Contract Workers entry from MTF Gate

- Contractor workers for MTF, Port and Jetty will enter through the MTF Gate.

- They would pass through turnstiles using access card with Hand Geometry reader.

F. Contract Workers entry into Haul Road Gate

- Contractor workers for carrying out repair / maintenance jobs within the Haul Road Area will enter through the Haul Road Gate on SH25.

- They would be allowed entry after checking their ID Card and permit to work in the Haul Road.

G. Contract Workers entry into Tunnel

- Contractor workers for carrying out repair / maintenance jobs inside the Tunnel will enter through the gate on the South side.

- They would be allowed entry after checking their ID Card and permit to work in the Tunnel.

## EXIT

- For exit, the Contract Workers would follow the above process in reverse.

- Random frisking / checking will be carried out at all the gates.