- RTF – MTF Pipeway Crossing (Nani-Khavdi Crossing)

Nani-Khavdi level-crossing on pipeway to be secured by a wall on both sides. The walls must be robust and tall (say 10’). Suitable intrusion detection system to be provided on these walls.

Traffic Density on RTF – MTF Pipeway corridor is expected to be as below

4 buses/shift

6 cars/shift

Single lane of 4.5 m wide for access control at gates is adequate.

Access control only by gate is adequate. No barriers/turnstiles are required.

Gates will be electrically operated.

Typical Haul Road Gate will have 15.5 m wide detachable section ( carbon steel grills cladded with CS sheets ) and a 4.5 m wide swing gate.

Quickly removable intrusion detection system will be mounted on top of this gate. Intrusion detection type to be recommended by ISDS.

There are 8 nos. such gates on Haul Road starting from zero point to SH 25 high way.

Nos.

Harbour Gate at Zero point		1

Salt Pan Gates				2

MTF South Gate				1

Corridor Gate				1

Corridor Gates				2

Corridor Gate				1

- Ro-Ro jetty has a Propylene Unloading facility. Local intrusion detection system shall be provided for this. Control and communication of this system shall be from MTF security control room.

No other portion of the ro-ro, lo-lo jetty is planned to be protected by any intrusion detection system. Thermic camera on the liquid jetty will be used for surveillance.

- Liquids Jetties

a) 10 nos. colour, 8x, PTZ cameras have been installed by Siapem. These are controlled from a local control room called POC (Port Operations Control )

These cameras to be also viewed from MTF control room. One monitor with 9 panels and one monitor for full view to be considered in MTF control room.

b) CCTV and access control system provided by Siapem to be integrated to MTF Control room.

c) Reliance to give layout drawings for jetty and approach trestle areas.

d) MTF to zero point haul road and the pipe sleeper corridor will be enclosed by a perimeter intrusion detection system.

e) Method for protection of water way under the approach trestle bridge.

Electronic Grill of approx. 10 m Height x 150 m long will be extremely expensive.

VMD on water may not be effective – false alarms due to water waves

Structural grill without any alarm will also be expensive as there is no ground support available. Bridge pillars are spanned approx. 60 m.

Following is concluded

No alarm producing fence to be provided.

Security Lights to be provided under the bridge.

Thermic camera to be used for surveillance.

If required in future, earthen bund may have to be provided below the bridge leaving I or 2 spans open for passage of water and a suitable fence can be installed on the bund.

- Thermic Camera Mounting Location

The camera needs to be mounted on a tall structure on the port with clear view of SPMs, pipe sleepers and knucle point. Hence it requires 360 degree movement.

Feasibility of mounting this on a 10 m tall tower on the Port Control Room to be explored.

Thermic camera will be controlled from the Port Control room. Viewing from MTF control room is also required (check?)

Cooler of the thermic camera can be inside or outdoor (?)

Programming of thermic camera for planned views

# Point no-11

## Only propylene facility will be protected no other system

Cameras on jetty –operated by port operations control –PIT12 colour explosive proof housing.

- Security to have 1 matrix VDU and second VDU for blow up.

- Only vieul (no control)

- Link between POC to MTF control room

- Security guard on talk to POC and operate PTZ.

- Shore link (0 pont) bridge-taut wire –ISDS

- RPL to give layout chg for jetty (trestle-Bridge)

- BOQ for all equipment –Base Qty antables brought by ISDS

# Point no-5

Labour colony –2 approx. 2000 contract workers staying with family

- Skilled workers for steady state

- Construction labours

- Permanent

- Entry-30minutes

- Iris control is recommended over hand geometry (problems dirt, cut, bandage)

- Pins

### HSP/KSS/CD/MPY/HHP

- one super camera for MTF

Indoor security-MAB

1200 liq. Trucks

200 sulpher

300 ep

100 MMC

100/150 marketing

200  coke

200 limestone

Registration of truck Nos. at  material gate.

- Vehicle not allowed 10 park which are not registered

- Flow chart & procedure document –VV

- Digital 1 number recounding signs option to be offered do

- Pack 200 trucks 1 hrs.

- Loud speaker / PA system for material gate.

- Exit anti pan back-no entry –no exit

- Separate lane for visitor (passenger) coves.

- Exit-truck /tanker is stopped checked for gate pass invoice and then allowed to go.

- Five minutes per truck to check

Stutters

Security

- Probability of truck/tanker cay explosive jey very less

- Dogs & electronics snifters –portable gasoline 	mix-up with explorers-difficult.

- Mobile explosive tracer - orderly ordered

- Separate random checking area

- Roof with Good lighting –overhead bridge.

- Ditech to check underneath the truck.

100 trucks per hrs.

5 minutes per truck

200 persons per hour-Biometrics

80% driver/tanker regular

- Isolated sector in parking –10 registered need personal & biometrics to genet a stop-make-touchkey.

- No link between driver/truck and touch key.

- Exit roule for rejected truck-not allowed to park in parking area.

- Three emergency gates open to ave-k

- Liquid parking out 2 bariers

Electrical operated by security guard

Two sli manual gates.

- Return of not announced tankers- U turn lane created –one manual sudis gate

## Two block ages

- Rejected truck goes back to ave-J

- Not announced with truck to go to liq. Gan

- Gate with sys-rejected truck to ave-j

- One 4.5 mk lane opening gate with removable structure-swing.

- Co-ordinates from SKS

Avenue-G1-two emergency gates in Liq. Gantry area. 6 mtr each.

- Road loading area-40 RPL employees

Per shift		-30 oil companies

Occupancy		-50  contract woud..

- Liq. Out gate location to be shifted UMD / 	Sptem

- Marketing

Rejected note and corf truck to ave-J

DG

Gate –9B –4.5 mtr as swinging gate with removable structure.

System -  out tryger with microphinic

- outtryger with taut wire.

- Interred detectors

RG 9C-swing Gate with taut wire.

RG-9 times nos. Manuel slicty gates.

Emergency – gates-three nos.

- Opening inside & outside

# Point no.1	 Train checking

- Probabilities of explosive sabotage very low.

- Elec. Sniffers-3000-4000 JSH 1 pieces too time consuming.

- Dog sriffers –recommended.

- Ditch under the Rail track.

- 2 mobile units.

#### Sinister areas :-

- CMC

- MAB

- MMC

- Spheres

Taut wire with gate (200 mtr) and wicked gate.

High fence with concerting

UMD –100 mtr alarm

80 mtr identity

60 mtr recognition

- Alarm light –PA System / siren.

- Breather windows-Microphonic cable running all over covering door

Prior information to control room

Door with magnetic sensor.

a. 20 kms	-10-15 US per mtr.

- CMC & MMC	-16 UMD\

No special lighting rear.

- Tunnel-pipe entry

- Grill –culvert –4.5 mtr swing

- Manual sating gate with taut wire system -NC-opening inside.

- Car sensor with green /red light on both sides Boom  Barier with pra

- Scheme to incorporate-in monsoon protection shed.

- Cryogenic tanks:

Chain link fence with barbed were (oil on top.

- Strong room-guns-Ammunition

- Finance-cash value

- Medical files

- ATM- b

- PIB/RCB/ACB/- no access control internal

# Access control philosophy

- Main gate:

- Boom barrier. Electrically operated by remote control.

- Spead breakers –lights recommended

- Location of boom bar.

- Security point of vial-no much concern

- Architectural e

- Turnstile- two no-S South of Bld.

- Metal detetecter (walk through)

- Visitors returning –turnstile should take the card back (M/c eat the could)

- Face of the company –security -	colater etc.

ISSWS

- Checking train for explosive , sabotage

- RTF-MTF pipe corridor –crossing for the village (two nos)

- Checking trucks /tanker –driver

- Long range camera for Refinery area (two nos)-helicopter tour.

- Labour colony from(separating)

Refinery site-

- Security options to be offered to Township residents.

- Maine area taut wire protection boundary & interface with thermal

- Gates 	–architectural/lay-out

-Traffic density

-Operation methodology

- Cuatter outlet panage, location, cross seet area

- Perimeter lighting & emergency lighting power shredding.

- Sensitive area like PIB , Tank farm , Breather , Guns Ammunition , MMC

- Open sea area-between fence road RO-RO Jetty –Fisherman.-any system

- General Access control philosophy method of operation , e.g. Bio-metric

- Township employees to refinery site –Security check

- Budgetary estimate

- Sizing & layout of control room (recommendation man power req.)

- Access control & CQTV system at liquid jetty.

- Perimeter protection line borders.

- Access Control enough buses

- ISDS suggested to build a tunnel or bridge-at Nani khavdi or side wall over pipe rack.

One guard manningtwo gates.

A buses –25 cars a day.

- Budgetery estimate does not include stainless buoyed wire.

- Haul road entry 1,2 corridor 2 Nani khavdi gates, 2 sait pan gates and 1 RO-RO Jetty gate.