Roles of security in On-Site Emergency Plan

- Introduction

In spite of sound design and engineering methods if proper care is not taken during operation, maintenance and inspection, a refinery can pose major threat to its employees, property and public due to major fire, explosion. Toxic release, oil spill etc. In other words a Major Emergency is disaster which the management of the area involved (incident site) can not control with the resources normally available to them and which threatens large numbers of people, area of plant for safety and health of public. Most of the emergencies can be controlled by careful evaluation of the anticipation of possible events and situations.

- Objectives

The overall objectives of MEMP are as follows:

- To contain and control emergencies.

- To prevent loss of life and minimize the risk of bodily injury to employees and neighboring population

- To minimize the damage to company installations and public property.

- To minimize impact on environment.

- To provide maximum possible safety for the emergency response personnel.

- To inform employees, public and authorities about the hazard/risk assessed safeguards provided and the role to be played by RPL.

- Emergency Scenarios

  - An Explosion

Explosion is a major hazard and the violence of explosion depends on the rate at which energy is released. Explosion can bring in certain effects like blast wave, thermal effects, missile effects, ground shocks and injury to personnel.

The explosion types considered for the purpose of Maximum Credible Accident (MCA) Analysis include unconfined vapor cloud Explosion (UVCE) and boiling liquid Expanding Vapor Explosion (BLEVE).

    - Maximum Credible Accident (MCA) Analysis

Maximum Credible Accident Analysis is a tool to find the maximum potential damage an accident can cause. It helps in the identification of most vulnerable sections of the plant and facilities that pose maximum possible damage if the hazardous materials are released as a result of a failure. Essentially this analysis will identify the damage situations that arise from heat radiation through fire, over pressure due to shock wave from vapor cloud explosion and toxic effects. The following accident scenarios were taken up for MCA Analysis:

    - BLEVE

A boiling Liquid Expanding Vapor Explosion occurs when there is a sudden loss of containment of pressure vessel containing super heated liquid or liquefied gas. The primary cause is usually an external flame impinging on the shell of a vessel above the liquid level, weakening the container and leading to sudden rupture of the shell. A pressure relief valve does not protect against this mode of failure. It should be noted. However that a BLEVE can occur due to any mechanism that results in the sudden failure of containment allowing a super heated liquid to flash and increasing its volume over several times. This is sufficient to generate a pressure wave causing fragments of the container flying distances apart. If the released liquid is flammable, a fireball may result. The most known type of BLEVE involves LPG. Once a fire impinges on the shell above the liquid level, the vessel fails within few minutes.

    - Unconfined Vapor Cloud Explosion

When a flammable gas or vapor escapes into the atmosphere and mixes with air, an explosive gas or vapor cloud can occur. This flammable mixture will explode if ignited, causing extensive damage to the surroundings.

    - Pool Fires

Pool fires are common fires resulting from fire over pools of liquid e.g. a storage tank, a bund etc.

    - Jet Fire

There is a variety of situations in which a jet flames, or ejected flame, can occur in the process industries, either by design or by accident. Ejection of flammable liquid from a vessel, pipe or pipe flange can give rise to a jet flame if material ignites. The jet flame from pressure relief valve may cause adjacent vessels to overheat and burst, giving a boiling liquid expanding vapor explosion.

    - Toxic Dispersion

The hazard presented by toxic chemical depends on the condition of exposure and the type of chemical. A very large release of toxic chemical such as Chlorine or Hydrogen sulfide under the most unfavorable conditions is usually regarded as having a disaster potential greater than that of fir or explosion.

  - Major Fire

Fire is a major hazard and is normally regarded as having a disaster potential less than explosion or toxic release that can be controlled at the incipient stage, Certain examples like a fire involving a storage tank is serious and would require several days of fire fighting.

  - Toxic Release

Release of toxic chemicals in the form of gases and vapor can pose major hazards if proper care is not taken. Such hazard presented by a toxic release depends on the duration of exposure and on the chemical itself. It ranges from a sudden brief exposure at high concentration to prolonged exposure at lower concentration over a working lifetime.

  - Oil Spill

Spill may occur on land or on water. Spills on land are relatively simple to handle. They normally require collection and transfer of oil and clean up of oily mud. Special attention is given to prevent the pollution of watercourse and aquifers spill on surface water is more difficult to handle and requires immediate emergency response.

  - Cyclone

As this refinery is located in the coastal area of gulf of Kutch, the possibility of cyclone striking the area cannot be ruled out. While establishing the strategic actions during such scenarios, factors such as poor communication, loss of utilities, disrupted logistic and life threatening rescue facilities are considered.

  - Earth Quake

An earthquake can pose a major threat to the plant, property, life and environment.

  - Air Raid

As this project is in close proximity to Pakistan, the possibility of air raid cannot be ruled out.

  - Product / Transport Emergency

During transportation of product, emergencies like Explosion, Fire, Leakage, Spillage or Toxic release are possible. It may be outside the factory premises, at nearby or far locations.

4.0      Means of communication

- Radios

- Telephones

- Satellite communication

- Heat / Smoke Detectors

- Breaking of MCP (Manual call points)

- Plant Telephones

- PA (Personal announcement) system

- Siren

- Radio Paging

- Messengers

The sense of urgency that every one experiences during emergency may lead to a chaotic situation if the mixture of communication systems do not function properly. It should be brief short and simple.

5.0	Emergency Response / General instructions.

Once emergency is declared, all the personnel inside the refinery including visitors shall respond in the following manner:

- The SS of respective plant / area shall assume the charge of incident controller till the arrival of Site Shift Manager (Incident controller) at emergency site. The incident controller will have a green band on his helmet as a distinctive identification.

- The plant personnel shall contact immediately their respective plant control room and shall proceed to safe assembly point based on the announcement made through P.A. system.

- All the vehicles operators shall pull their vehicles to the left and stop the engines. Engine keys shall be left inside the cabin itself.

- Visitors shall be necessarily accompanied by the Reliance employees and guide to a safe location as per the announcements.

- In case of toxic release proceed to the nearest building in the crosswind direction and close all doors and windows. Ventilation system like air coolers and air conditioners shall be switched off.

- If the gas or vapor suspected to have entered the structure, employees shall hold a wet cloth over nose and mouth.

- If the situation escalates beyond control, employees shall evacuate the site as per the instruction made through PA system

- Response of Security Zone / Area Officer

- On receipt of the Emergency call, Zone / Area officer to respond immediately to the emergency after confirming its nature.

- Mobilize maximum available security manpower, Minimize the use of vehicles, stop all the vehicles at safe distance, otherwise security vehicles may add the fuel in the scene.  Control Room officer to follow Emergency procedure.

- While making turnout, prevailing wind direction should be checked and make the approach to the site accordingly.

- Report to Shift supt. / Incident controller / SSM and act as security coordinator till time next Senior Security executives report at the site, Mobilize security personnel, Control the traffic, Evacuate the personnel not attending emergencies to safe place/ Assembly points.

- Control access to and ingress from the site to Prevent entry of unauthorized persons. While same time allow the Emergency services to enter and leave freely.

- Cardon-off Avenues and Streets around the incident site and establish alternate routes.

- All the Emergency services should be directed at upwind direction of the Emergency site ( as per the situation )

- All types of works/hotwork/coldwork shall be stopped.

- Make sure that Persons/s attending emergencies are wearing Protective gears, Suites, BA sets etc as required.

- Assist the casualty evacuation, Fire personnel.

- Assist as Material co-ordinator by opening the store (after the working hours for emergency material ) as directed by SSM

- Other relevant action should be taken depending upon the situation, in consultation with Incident Controller/SSM.

Coordinate with Security Control Room for Mobilizing additional security force, Assistant from other Zones / Areas or from Residence.

All the security personnel to wear safety Shoes, Helmet, Safety goggle (Suit combined if possible).

- Response of Security Control Room

- On receipt of any emergency, call alert Fire, SSM, Zone/Area security officer, Medical etc.

- Call the essential persons to establish on site Emergency Organization.

- Act as security coordinator till time arrival of senior security personal at incident site,

- Mobilize additional security force, depute security staff for regulating access and evacuation and give direction to prevent unauthorized entry into plant and control gathering of people.

- Ensure manning of the security gates, which are normally unmanned but to be used for evacuation during emergency.

- Deploy security Guard at CCP, RV, H&PP and hospital etc, as per requirement.

- Liaise with local police, nearest Fire stations and other Government agencies as per the instruction from CEC/Incident controller and mobilize additional security force, Auxiliary Fire squad etc, if required.

- Ensure vigilance at all security posts / Gates and be in constant contact with Site security coordinator /SS/ SSM and other members connected with incident/Fire fighting and rescue operation.

- If available, detail a separate officer to monitor the Radio Communication, and Maintain chenorological record of emergency action.

- Security coordinator

- Report to incident controller.

- Take charge of all security functions like mobilization of security personnel, threat analysis, traffic control, evacuation of personnel etc.

- Depute security staff for manning gates and incident site.

- Give directions to prevent unauthorized entry into the plant and control gathering of people.

- Guide evacuation of persons as per guidance of the incident controller.

- Ensure manning of the security gates, which are normally unmanned but to be used for evacuation during emergency.

- Liaise with local police under instruction from CEC and mobilize additional security force if required.

- Depute security officer to act as radio officer in the CECR to log all the incoming and outgoing messages and important emergency actions.

- Communicate with the Makers Control Room and top Management functionaries.

- Establish command posts at Refinery main entrance, incident site, Tank Farm, Safe assembly points and other locations as appropriate.

- Cardon-off Avenue and Streets around the incident site and establish alternate routes.

- Liaise with Reliance group companies to secure additional manpower.

- Central Emergency Control Room (CECR)

The CECR is located in Main Fire Station. It is a fixed location, which is safe and reasonably, remote from major processing area from where the widespread consequences during emergency can be better controlled. The Chief Emergency Coordinator takes charge at the CECR and command the emergency operation.

Mobile Emergency Control Center (MECC)

The Mobile Emergency Control Center is a vehicle accommodating all relevant information, necessary hardware, software, items required to manage an emergency at any place inside RPL complex.

Siren Tones

The actuation of siren varies with regard to the incident nature like fire and Flammable Gas/Toxic release and Air raid. Following are the various configurations set for different types of emergency scenarios:

| Sl.No | Type of Emergency | Siren Tone |
| --- | --- | --- |
| 1 | Major fire/Flammable gas release | Wailing tone for 2 minutes (8 times) 10 sec-on, 5 sec-off, 10 sec-on, 5 sec-off. |
| 2 | Toxic release | Wailing type siren for 2.5 minutes (6 times) 20 sec-on, 5 sec-off, 20 sec-on, 5 sec-off |
| 3 | Air Raid Warning | Wailing type siren for 6 minutes (10 times) 30 sec-on, 5 sec-off, 30 sec-on, 5 sec-off |
| 4 | All Clear / Testing | Continues siren tone for 2 minutes. |

Halt and Park / Proceed Point (H&PP)

Purpose: - For Fire and Ambulance personnel to assemble prior to being directed forward to the incident and for them to regroup between spells of activities.

Duties:  -

- Restrict movement of unwanted Vehicles.

- Traffic control for easy movement of Fire, Ambulance and other emergency vehicles.

- Mob control, Regulating personnel as directed by controlling agency.

- To keep the Avenues and Streets Clear.

Causality Collection Point (CCP)

Purpose: - A safe location for diagnosis of causalities from incident site before dispatch for further treatment. An area which is safe and readily assessable by Ambulance.

Duties; -

- Mob control, only authorized personnel to be allowed.

- Traffic control to enable free movement of Ambulance or other Emergency vehicles carrying causalities.

- Assist medical personnel in causality evacuation movement if needed.

Rendezvous Point (RV)

Purpose: -Rendezvous area is available to Police, Fire, or Ambulance to report on arrival or when requiring further instructions.

Duties: -

- Prevent assess to and ingress from the site to flow the Emergency services to enter and leave freely.

- Direct local authorities/press to reception.

- Direct outside Fire tenders / Ambulance to H&PP.

Assembly Points

Purpose: - To assemble plant personnel those who are not responding emergency at safe place.

Duties: -

- Do not allow to rush persons to the scene to be a spectator.

- Direct guest/visitors/construction employees toward company gates if desired.

- Remain alert for announcement from Plant Control Room.