Ü In reply please quote: Ref. JG/4296

04 August 1999

RELIANCE PETROLEUM LIMITED

JAMNAGAR INDIA

ATT: Mr. A. K. CHAUDHRY

General Manager (Procurement)

Dear Sir,

It is our pleasure to give you herewith our best proposal for a complete, turn-key and comprehensive Perimeter Intrusion Detection System around your site. As this site is a high security site, we propose to install our DTR-2000 Taut Wire Intrusion Detection System (TWIDS), as its main component, and this for the following reason:

Magal has been designing, manufacturing and installing the DTR system for almost 30 years. During that time we have installed around 5,000 kilometers of TWIDS world wide, in all possible locations, terrain and climatic conditions. From the sub-zero climate of Northern Canada to the tropical humidity of Sri-Lanka – our systems perform well and give reliable service in any weather condition.

The principle of operation of the DTR-2000 is simple, and thus highly reliable. It is based on an Electro-mechanical “switch”, completely sealed, which, when activated, gives a signal to the CCC (Computerized Control Center) to produce an alarm.

The DTR-2000 fence itself consists of a set of closely spaced wires (we suggest that barbed wires be used) which are mounted between posts (Sensor Posts and Anchor Posts, explained below) and stretched tightly between them. The pre-tensioned wires are connected to the sensors (see below), and, as the tension on both sides of the sensor is the same, do not exert any force on it. If however this equilibrium is disturbed the internal contact points of the sensor will close and cause an alarm.

Any attempt to penetrate the site by climbing, spreading, cutting the wires or leaning a ladder against them is immediately detected by the system, which notifies the guard in the Control Room that such an attempt is taking place. The system not only give this indication in real time, but gives information about the exact location of the attempt, whether the intruder is climbing the fence, pulling on it to spread the wires, cutting them or trying to climb over by means of a ladder.

The most important advantages of the DTR-2000 system are:

- It is the most reliable, trouble free and dependable system available anywhere today. The Probability of Detection (P.o.D.) of the DTR-2000 is in the 97 – 98% rate.

- It is the system having the lowest False Alarm Rate (FAR) of any system available on the market today. Correctly installed, it is guaranteed to give less than One False Alarm per Kilometer of System per three months. No other system can claim – and deliver – such a performance.

- The system is completely automatic, and does not depend on human alertness.

- The maintenance of the system is simple, and requires very little skill or time.

- It is suitable or any terrain or weather condition

- And, last but certainly no least, it is manufactured by a Company with more experience, more installations and more engineering and technical know-how than any other company worldwide.

As mentioned, these Taut Wires are firmly connected to the Anchor Posts, located about 50 meters apart. Halfway between them the Sensor Posts are placed, on which the Sensors mentioned above are located, and to which the Taut Wires are firmly connected.

A short Technical Description of the Systems and their main components is given herewith. Further information can of course be obtained from the additional literature enclosed.

#### Technical Description – DTR-2000

### Sensor Post

The Sensor Post, installed on an average of 50 meters apart, consists of a heavily anodized Aluminum profile for the back part, as well as a cover made of the same material. The two parts are held together by means of special micro-switch-equipped tamper proof clips, assuring that the cover cannot be removed (and the sensors tampered with) without giving an alarm.

The special, proprietary DTR-2000 sensors are mounted inside this post. Two each of the Taut Wires are firmly connected to each sensor. Thus, on a free standing 28-wire system (straight or with an “outrigger”) the post will include 14 sensors, on a 12 wire wall mounted system there will be 6 sensors on each post .

The sensor itself the heart of the DTR system, is constructed of weather resistant materials, (Lexan, Stainless Steel, EPDM and gold plated contacts, completely sealed). A pull on any of the barbed wires (above a certain pre-set minimal limit) caused by pulling, spreading, cutting, or climbing the wires will activate an immediate alarm in the CCC.

As in most applications, the system planned for this project will have an “outrigger” configuration. The upper part of the post, nominally one meter long, is connected to the 2 meter vertical free standing part (or to the short wall mounted section) at a 45 or 60 degree angle, in order to “give depth” to the system, and thus increase its performance.

The SPRU (Sensor Post Reporting Unit) is also mounted inside the Sensor Post housing. This unit, completely weather protected, (potted) serves as the communication element between the Sensor Post and the CCC, the Computerized Control Center.

Alternatively, we may use the GPRU Unit, mounted externally,  on a group of sensor posts, depending on final design.

### Anchor Post

The Anchor Post, which serves to hold the pre-tensioned Taut Wires in place, is made of a special Steel Profile, heavily galvanized (In the “corrosion resistant” version, also quoted herein, this profile is supplied in Stainless Steel) and suited to withstand the tensions placed on it by the Taut Wires. Inside this section, the “Breakable Plates” are bolted. These plate serve to hold the Taut Wires firmly in place, they will however break (and thus release the Taut Wire, causing an alarm) if a vertical force, greater than 20 Kg (as by a climber) is exerted on them.

### System Configuration

Because of the different fence configurations around the site, we are going to install our DTR-2000 system in 2 main configurations:

- A Free Standing system, constructed on a carrying fence which will be erected on a continuous concrete foundation (concrete strip). The height of this system is 2 + 1 meter (nominal), having 20 + 8 wires

- A wall mounted system, bolted onto either a concrete or a pre-cast wall. The vertical length will be 40 cm, and the outrigger 1 meter (nominal) This configuration will have 4 + 8 wires.

### Spirals

In order to keep the sensitivity of the system constant throughout its whole length, special movement limiters – in the form of spirals are installed all along the system. These spirals, (of two types, “Main” spirals and “Intermediate” spirals) allow the free and unlimited movement of the Taut Wires along their length, but transfer any movement perpendicular to that direction (vertical or horizontal) into longitudinal movement – which activates the sensor and thus causes an alarm)

### Communication Cable

The alarm signals, as well as the constant “dialogue” between the Sensor Posts and the CCC is conducted through a highly UV resistant, double coated four twisted pairs cable. This cable runs all along the fence length, easily accessible for maintenance and repair. Any cut in this cable, or any attempts to short it, will cause an immediate alarm. Additional cable is required between the location of the CCC and the nearest point on the fence. (Communication between the CCC and the SMS (See below) will be through existing fiber-optic cables)

### Barbed Wires

The “standard” barbed wire supplied by Magal is a specially coated and constructed double strand wire, termed “BEZINOL” It is suitable for most applications where chemical and atmospheric corrosion exists

In extreme cases, such as the conditions above but where the installation is very near the sea, and where the corrosive atmosphere is extreme, Magal installs Stainless Steel (304) barbed wire.

### CCC

### The CCC (Computerized Control Center) is based on software written by Magal, and a special Magal PCB inserted in a commercially available hardware. (A standard IBM compatible PC, Pentium II) Each such card has the ability to communicate with 256 “addresses”, and, if needed, two cards can be installed in each PC. Thus, on the average, each such PCP is used for a 5 kilometer perimeter and will include two PCBs

The price of these CCCs is included in the DTR-2000 system price.

Notes:

In order to connect the CCC to the SMS an additional card, a TCP/IP card, will have to be used.

#### Technical Description – Other Systems

### Electric Grids

In order to prevent intruders from entering the site through various openings such as water and other pipes, Magal will install MAGBAR units on those. Any attempt to penetrate through the Magbar by trying to bend the bars, by cutting through them, will generate an alarm. A magnetic switch incorporated in these units, will give an alarm signal if an attempt is made to lift or otherwise remove the MAGBAR from their location .A leaflet giving additional details is enclosed

Note:

In order to communicate with the CCC, the Magbar requires an SPRU or GPRU Unit, depending on detailed design. If two Magbars are within a short distance to one another (say within 100 meters or so) they will be considered as ONE “address” in the CCC and one SPRU unit will be sufficient. If more than two units are adjacent, (up to a total of 7), we will install our GPRU Unit, at a cost of $ 1,100. Needless to say, the GPRU is supplied tamper proof.

### Gate Protection

Three main types of gates exist on this site:

- Sliding gate

- One wind swinging gate

- Two wing swinging gate

All gates will be equipped with a special “Gate Configuration” of the DTR-2000 system. The prices given in our quotation include a suitable communication unit for each gate

Our quotation give the prices for both the Bezinol as well as the Stainless Steel version.

### Piezo Electric Intrusion Detection System

This easily installed system consists of a Piezo Electric cable, which will be mounted primarily on the pre-cast walls. It will use the communication cable of the Taut Wire System, but will give separate indication of an alarm. Its purpose is to give an alarm if someone is trying to intrude into the site through, rather than over this fence. The material cost for this system is $ 15 per meter.

### Active Infra Red (IR) Units

Three types of IR units are being offered:

- A 1 meter high one, having two double IR units

- A 2 meter high one, having four double IR units

- A 3 meter high one, having five double IR units

In order to prevent over-climbing of these element, they will be equipped with a special ”cover cup” designed to give an alarm should such an attempt take place.

Kindly see the enclosed leaflets for additional information.

### Panic Buttons and Guard Patrol

At specified points on or near the Perimeter, we will install push buttons as a “Guard Control” system. These points have to be activated by the guard at certain pre-determined times. Should the guard NOT activate those points within the time period specified, an alarm will be given.

Guards will also have the option to use Panic Buttons, which they may activate and alarm the CCC that some unplanned event is taking place.

### Floodlights and Sirens

At certain critical points, we will install Floodlights and/or Sirens. (at a cost of US$ 320 each)  These will be activated by the CCC when, and only when, an alarm from the fence unit has occurred Each such unit will require a 220VAC/24VDC unit, and will be equipped with a GPRU unit as needed.

SMS (Security Management System) MagNet

The MagNet system is a network that integrates management, control and display of various different security systems to a single real-time database. The system is based on computer communication that is controlled by a single unique server developed by Magal. The field units convert the sensor and detector data to the network language. The server converts in real time data to the database fields, analyses the data and transmits operational commands accordingly. The operational commands can be sent either back to the field units or to the operator workstations. The purpose of the workstations is to convert the server decisions to visual information and to allow the operator to respond and influence system operation.

The MagNet allows a single operator or a number of operators to simultaneously control some of the systems or all of the systems in the network, according to authorization. The system can be entered from anywhere in the world using the Internet as long as the appropriate authorization exists. The MagNet is not limited to the number of operators, number of security systems connected or to the distance between the network components. The use of Ethernet boards and the TCP/IP protocol allows the use of a wide range of communication media such as: telephone lines, fiber optics, wireless communication, etc. The MagNet integrates various detection technologies (infrared, microwave, etc.) with the display of video pictures of the alarm source using an advanced video matrix for the network. The MagNet is a system, which receives the alarms and provides explanation as to the cause of the alarm.

In order to effectively control all the security systems on this site, Magal will install this MagNet Security Management System. Thus, all security systems will be integrated under one unit. The communication between the CCC and the SMC will be by means of existing Fiber-Optical Cables using LAN configuration and TCP/IP protocol.

Description of Taut Wire configurations supplied:

(See enclosed drawings # 7021-3839 & 3840)

For the particular site which we have inspected, we therefore submit to you our best offer as follows: (cost given for 1 km’ of fence)

1. A free standing DTR-2000 system, Approx. 2 meters vertical with 10 sensors (20 wires) and an approximately 1 m’ outrigger (4 sensors, 8 wires.)

Cost of equipment for this configuration, FOB Magal	See enclosed table

2. A free standing DTR-2000 system, Approx. 2 meters vertical with 10 sensors (20 stainless Steel wires) and an approximately 1 m’ outrigger (4 sensors, 8 wires.), but for locations in a highly corrosive atmosphere, being constructed of special, highly corrosion resistant materials (Stainless Steel 304, Specially coated Aluminum, etc.)

Cost of equipment for this configuration, FOB Magal	See enclosed table

3. A pre-cast wall mounted system, having a short (~40 cm) vertical section, with 2 sensors (4 wires) and the same outriggers as the free standing one.

Cost of equipment for this configuration, FOB Magal	See enclosed table

4. The same system (para. 3), but specially constructed for a highly corrosive atmosphere	See enclosed table

5. The same system (para. 3), but wall mounted	See enclosed table

6. The same system (para. 5), but specially constructed for a highly corrosive atmosphere	See enclosed table

Detailed design and specification of System		% of total equipment cost

System testing, final customer acceptance & documentation		% of total equipment cost

Management fee		% of total equipment cost

Per Diem cost for a Magal Engineer:

On Site (Including traveling days)

In Israel

Flight costs, room and board and transportation to and from site to be born by you.

Note:

On all work connected with our system but performed by an outside contractor Magal  will charge a  % inspection and warranty fee.

Delivery	 during 5 Months after receipt of order and all payment conditions

Validity: This offer is valid until August 30 1999.

We hope we have given you all the necessary information, should you require any additional details, we are at your service.

Very Truly Yours

B. Brimer

Head of Project Department

MAGAL Security Systems Ltd.

ISRAEL

Enc.:

Magal Table of Organization

Drawing # 7021 – 3840

Drawing 7021 – 3839

Detailed Cost Estimate

Catalog Pages of equipment (Most of the catalogs have been handed over)

Appendix

Referring to your letter of August 3rd, herewith our comments:

Scope of work and exclusions:

Given above

Datasheets etc

Enclosed herewith

Indicative Layouts and size for main control room etc..

As we are at present in the process of designing the Perimeter System only, this design will be included at a later stage

Plan for engineering and software development

Will be given with detailed system design

Project implementation plan

Enclosed herewith

Procurement Plan

Major Items for Perimeter System are manufactured by Magal. Subvendors for additional equipment will be chosen after detailed design completed

Quality assurance and Quality control plan

CE and ISO certificates were submitted to you. Detailed plans will be submitted

Proposed deployment of recourses

This point is unclear. Please clarify

Requirements from the owners

The following will be supplied by the owner:

An office  suitable for 3 people

Telephone with International Direct Dialing

Fax Machine

Computer incl. Internet and connection to your printers

Car

Secretarial Services

Draftsman conversant in ACAD

Guarantee/Warranty Period/Maintenance Philosophy

Magal’s standard warranty period is for a period of 12 months for all equipment supplied by us. After that period, we will supply a system warranty (not including willful damage) at a yearly cost of 4% without spare parts, and 6% including spare parts. A standard service contract will be submitted to you for your consideration

List of spare parts

Will be given with detailed system design

Training

Your personnel will be fully trained by us during the installation period in operation, maintenance and repair of the system